SECTION 09 65 00 - RESILIENT FLOORING

PART 1 - GENERAL

- SUMMARY

- Section Includes:

- Luxury vinyl tile flooring.

- Luxury vinyl plank flooring.

- Rubber tile flooring.

- Resilient base and accessories.

- Related Sections:

- Section 03 30 00 - Cast-in-Place Concrete.

- Section 09 05 61 - Common Work Results for Flooring Preparation.

- REFERENCES

- Reference Standards:

- ASTM F1700 - Standard Specification for Solid Vinyl Floor Tile.

- ASTM F1861 - Standard Specification for Resilient Wall Base.

- ASTM F710 - Standard Practice for Preparing Concrete Floors to Receive Resilient Flooring.

- ASTM F2170 - Standard Test Method for Determining Relative Humidity in Concrete Floor Slabs Using in situ Probes.

- SUBMITTALS

- Product Data: Submit manufacturer's technical data for each flooring product.

- Include wear layer thickness and overall thickness.

- Include adhesive recommendations and limitations.

- Samples: Submit two full-size samples of each flooring type and color.

- Maintenance Data: Submit cleaning and maintenance instructions for Owner's manual.

- DELIVERY AND STORAGE

- Deliver materials in manufacturer's original unopened packaging.

- Store materials in conditioned space at 65 to 85 degrees F for minimum 48 hours before installation.

PART 2 - PRODUCTS

- MANUFACTURERS

- Acceptable Manufacturers:

- Armstrong Flooring.

- Mannington Commercial.

- Tarkett.

- Shaw Contract.

- Substitutions: In accordance with Section 01 60 00.

- RESILIENT FLOORING

- Luxury Vinyl Tile (Type LVT-1):

- Size: 12 inches by 24 inches.

- Overall Thickness: 2.5 mm minimum.

- Wear Layer: 20 mil minimum.

- Application: General office areas and corridors.

- Color: As selected by Architect from manufacturer's standard range.

- Luxury Vinyl Plank (Type LVP-1):

- Size: 6 inches by 48 inches.

- Overall Thickness: 4.0 mm minimum.

- Wear Layer: 28 mil minimum.

- Application: Break rooms and conference rooms.

- Pattern: Wood grain.

- ACCESSORIES

- Resilient Wall Base: Rubber, 4 inches high, 1/8 inch thick.

- Style: Cove, top set.

- Color: To match flooring.

- Adhesives: As recommended by flooring manufacturer for specific application.

- Transition Strips: Aluminum with anodized finish, matching flooring height.

PART 3 - EXECUTION

- EXAMINATION

- Verify substrate is ready for flooring installation:

- Concrete is fully cured minimum 28 days.

- Moisture testing per ASTM F2170 shows 80 percent RH maximum.

- Surface is smooth, clean, and free of curing compounds.

- Floor flatness meets FF 25 minimum.

- Do not proceed until unsatisfactory conditions are corrected.

- INSTALLATION

- Install flooring per manufacturer's written instructions.

- Apply adhesive uniformly with proper trowel notch size.

- Observe open time requirements.

- Roll flooring with 100 pound roller after installation.

- Tightly butt joints without gaps or overlaps.

- Scribe flooring to walls and permanent fixtures.

- Install wall base after flooring is complete.

- Install transition strips at changes in flooring material or direction.

- CLEANING AND PROTECTION

- Clean flooring per manufacturer's recommendations.

- Protect installed flooring from construction traffic.

- Cover flooring with building paper or other approved protection until Substantial Completion.

END OF SECTION